| RFID Application Note # RFID Trigger Configuration & Event Subscription # Revisions REV; Description; Date; Author. 1.0.0; Initial Draft; Feb 17, 2026; Chuck Lin. |
| --- |
| ` |  |
| DOC NO: SPR-59555 | Rev 1.0.0 |

## 1. Overview: Hardware Trigger and Software Event Configuration

The RFIDHandler class manages the Zebra RFID reader's hardware trigger, which serves two distinct functions depending on the application state:

- RFID Mode: The trigger starts/stops RFID tag inventory.

- Barcode (Sled Scan) Mode: The trigger activates the barcode scanner.

To support these modes, the application could dynamically reconfigure the hardware key layout and manage event subscriptions. This document details the design of setTriggerEnabled (hardware config) and subscribeRfidTriggerEvents (software event masking).

## 2. Architecture & Synchronization

### 2.1. The Challenge

Reconfiguring the hardware trigger (setKeylayoutType) involves sending commands to the reader. If the reader is currently performing an inventory, read access or write command (busy) or if multiple threads attempt to configure it simultaneously, the SDK may throw exceptions (e.g., OperationFailureException) or the reader may enter an undefined state.

### 2.2. Synchronization Strategy

To ensure thread safety and hardware stability, we employ a "Wait-for-Idle" pattern protected by a ReentrantLock.

- resourceLock (ReentrantLock): Ensures mutual exclusion. Only one configuration operation can proceed at a time.

- bRfidBusy (volatile boolean): Acts as a guard. Configuration commands are blocked until this flag is false (indicating the reader is idle).

- waitForReaderIdle(): A helper method that polls bRfidBusy with a timeout mechanism to prevent deadlocks.

## 3. Detailed Design

### 3.1. setTriggerEnabled(boolean isRfidEnabled)

This method physically reconfigures the reader's hardware trigger behavior.

Workflow:

- Acquire Lock: Enters resourceLock to block other configuration attempts.

- Wait for Idle: Calls waitForReaderIdle().

  - If the reader is busy (scanning), it waits up to 2 seconds.

  - If the timeout is reached, it throws a TimeoutException and aborts to prevent hardware errors.

- Determine Mode:

  - true → ENUM_NEW_KEYLAYOUT_TYPE.RFID (Trigger starts RFID Inventory).

  - false → ENUM_NEW_KEYLAYOUT_TYPE.SLED_SCAN (Trigger activates Barcode Scanner).

- Apply Configuration: Calls reader.Config.setKeylayoutType(mode, mode).

- Release Lock: Ensures the lock is released in a finally block.

### 3.2. subscribeRfidTriggerEvents(boolean enable)

This method controls the software event for whether the application receives HANDHELD_TRIGGER_EVENT notifications from the SDK.

Purpose:

- Enable (true): The app receives callbacks when the trigger is pressed/released. This is essential for RFID mode to programmatically start/stop inventory (read, write or access commands).

- Disable (false): The app ignores trigger events. This is useful when switching to Barcode mode to prevent the RFID logic from reacting to trigger presses intended for the scanner.

## 4. Integration Workflow

When switching modes, these methods must be used in a specific sequence to ensure a clean transition.

### Scenario: Switching to Barcode Mode

- Unsubscribe Events: Call subscribeRfidTriggerEvents(false).

  - Reason: Stop the RFIDHandler from receiving "Pressed" events immediately, preventing it from trying to start an RFID inventory while we are reconfiguring.

- Reconfigure Hardware: Call setTriggerEnabled(false).

  - Reason: Tell the hardware that the trigger should now act as a scanner button.

### Scenario: Switching to RFID Mode

- Reconfigure Hardware: Call setTriggerEnabled(true).

  - Reason: Tell the hardware the trigger is for RFID.

- Subscribe Events: Call subscribeRfidTriggerEvents(true).

  - Reason: Now that the hardware is ready, the application start listening for press events to trigger performInventory().

5. Safety Mechanisms

## 6. Deadlock-Avoidance Contract

To keep trigger mode changes safe under concurrent events, the implementation follows these rules:

- Never block indefinitely while holding resourceLock: waitForReaderIdle() has bounded retries and fails fast with TimeoutException.

- Never require event thread locks for busy-state transitions: bRfidBusy is volatile and updated directly by status events.

- Always release lock on every path: setTriggerEnabled and restoreDefaultTriggerConfig unlock in finally.

- Switch order is intentional:

  - RFID→Barcode unsubscribes RFID trigger events before hardware switch.

  - Barcode→RFID applies hardware mode first, then subscribes to RFID trigger events.

## 7. Code Reference:

## 8. Implementation Guide: Step-by-Step

### 8.1 Initialize RFID Event Listeners

To properly manage the "Busy" state of the reader, the application must listen for INVENTORY_START_EVENT and INVENTORY_STOP_EVENT.

Implementation: In the RFID API configuration, enable the specific start and stop inventory events:

### 8.2 Manage Reader Busy State (bRfidBusy)

The application tracks whether the reader is currently scanning tags using a volatile boolean bRfidBusy. This prevents configuration changes while the radio is active.

Implementation: Update the flag inside the eventStatusNotify callback:

### 8.3 Handle Hardware Trigger Events

This logic determines how the application responds when the physical trigger is pressed while in RFID mode. It includes a check for bRfidBusy to prevent race conditions (duplicate commands).

Implementation: When a HANDHELD_TRIGGER_EVENT is received:

prevent race conditions (duplicate commands).

At MainActivity.java:

### 8.4 Workflow: Switching to Barcode Mode

When switching from RFID to Barcode mode, the order of operations is critical to prevent the RFID handler from interpreting the next button press as an inventory command.

- Unsubscribe Logic: Call subscribeRfidTriggerEvents(false).

- Hardware Configuration: Call setTriggerEnabled(false) to switch the hardware key layout to SLED_SCAN.

- Switch the hardware trigger config to Barcode: setTriggerEnable(false)

- Once received barcode data, restore to RFID: re-subscript the software enable = subsriptRfidTriggerEvent(true) and configure trigger to RFID  = setTriggerEnable(true)

### 8.5 Workflow: Restoring to RFID Mode

Once the barcode scan is complete, the application must restore the reader to RFID mode.

Implementation: Triggered via the barcodeData callback (UI thread).

Popup UI to prompt user to Pull Tigger and Scan a barcode, once the barcode data delivery to the app: pop up UI to confirm “Restore to RFID” and calls two methods:

- subscibeRfidTriggerEvents(ture) for software event

- setTriggerEnable(true) to configure the hardware trigger to RFID

### 8.6 Safe Orchestration Summary

To ensure thread safety and prevent deadlocks, the switch to Barcode mode is best triggered inside the INVENTORY_STOP_EVENT. This guarantees the radio is off and bRfidBusy is false before the configuration change begins.