# Trigger Sync Doc: bRfidBusy and Deadlock-Safe Trigger Switching

## Purpose

This document defines the runtime contract for safely switching the RFD40 trigger between RFID inventory and barcode scanning modes.

## Core Problem

The trigger is shared by two subsystems: - RFID inventory (reader.Actions.Inventory.perform/stop) - Barcode scanner pull trigger (DCSSDK_DEVICE_PULL_TRIGGER)

If the trigger mode is changed while RFID is still active, SDK calls can fail or become unstable.

## Concurrency Model

- Single worker thread: ExecutorService executor = Executors.newSingleThreadExecutor() for serialized operational tasks.

- Lock for trigger config: ReentrantLock resourceLock wraps trigger configuration methods.

- Busy state signal: volatile boolean bRfidBusy tracks RFID operation state from status events.

## Busy-State Lifecycle

- bRfidBusy = true on INVENTORY_START_EVENT

- bRfidBusy = false on INVENTORY_STOP_EVENT

This state is used as a guard before any trigger-layout switch.

## Deadlock-Safe Rules

- setTriggerEnabled(...) and restoreDefaultTriggerConfig() must always acquire resourceLock.

- Any lock acquisition path must release lock in finally.

- Waiting for idle must be bounded:

  - waitForReaderIdle() retries 10 times with 200ms sleep (max 2s).

  - Throws TimeoutException on timeout.

- No blocking dependency on event callbacks while lock is held.

- On timeout/failure, return false and keep system responsive.

## Safe Switch Sequences

### RFID → Barcode

- subsribeRfidTriggerEvents(false)

- setTriggerEnabled(false)

Why: Prevent RFID trigger callback handling while hardware keylayout is changing.

### Barcode → RFID

- setTriggerEnabled(true)

- subsribeRfidTriggerEvents(true)

Why: Ensure hardware behavior is ready before consuming RFID handheld trigger events.

## Failure Handling Contract

- If reader is disconnected/null: return false.

- If reader remains busy beyond timeout: return false and log timeout.

- If SDK throws (InvalidUsageException, OperationFailureException): return false and log error.

- Never leave resourceLock locked.

## Relevant Methods

- waitForReaderIdle()

- setTriggerEnabled(boolean isRfidEnabled)

- restoreDefaultTriggerConfig()

- subsribeRfidTriggerEvents(boolean bRfidHardwareTriggerEvent)

- eventStatusNotify(...) for inventory state transitions

## Verification Checklist

- Press trigger repeatedly during mode switch; app should not freeze.

- Force inventory active while switching; function should timeout safely and return false.

- Confirm lock release by attempting subsequent switch operations after a failure.

- Confirm RFID trigger events are ignored in barcode mode.

- Confirm trigger events resume in RFID mode after re-subscription.