# RFIDHandler Trigger State Management: Design Logic

## Overview

This document describes the contract and logic for managing the trigger state transitions between RFID and Barcode modes in the RFIDHandler class. It focuses on the use of the bRfidBusy and bSwitchFromRfidToBarcode flags, and the correct sequence for event subscription and hardware configuration.

## 1. bRfidBusy (Busy Guard)

Purpose: - Indicates if the RFID reader is currently busy (e.g., performing inventory or other operations).

Contract: - Set to true: - On INVENTORY_START_EVENT (RFID operation begins). - Set to false: - On INVENTORY_STOP_EVENT (RFID operation ends). - Threading: - Only updated by the event handler thread. - Usage: - Checked before configuration changes (e.g., setTriggerEnabled, restoreDefaultTriggerConfig). - If true, configuration is rejected and the UI is notified. - Debugging: - All state changes are logged for traceability.

## 2. bSwitchFromRfidToBarcode (Mode Switch Guard)

Purpose: - Controls transition of trigger event handling from RFID to Barcode mode.

Contract: - Set to true: - On INVENTORY_STOP_EVENT in test mode, before switching to barcode. - Set to false: - When switching back to RFID mode (in setTriggerEnabled(true)). - Usage: - Checked in the event handler for HANDHELD_TRIGGER_EVENT; if true, RFID trigger events are ignored. - Debugging: - All state changes are logged for traceability.

## 3. Event Subscription and Hardware Configuration

subscribeRfidHardwareTriggerEvents(boolean enable): - Enables or disables the app’s receipt of HANDHELD_TRIGGER_EVENT notifications. - Called in setTriggerEnabled and testBarcode to align software event handling with the current hardware mode.

setTriggerEnabled(boolean isRfidEnabled): - Physically reconfigures the reader’s hardware trigger behavior. - Uses resourceLock to ensure mutual exclusion. - Checks bRfidBusy and aborts if busy. - Sets the trigger mode based on isRfidEnabled. - Calls subscribeRfidHardwareTriggerEvents after successful configuration. - Resets bSwitchFromRfidToBarcode when switching back to RFID.

## 4. Test Mode Logic

When the application is in test mode (context.getTestStatus() == true): - On INVENTORY_STOP_EVENT, bSwitchFromRfidToBarcode is set to true and logged. - subscribeRfidHardwareTriggerEvents(false) is called to prevent RFID logic from handling trigger events. - The UI is notified to scan a barcode. - testBarcode() is called, which disables RFID trigger events and sets the hardware trigger to barcode mode. - When switching back to RFID mode (setTriggerEnabled(true)), bSwitchFromRfidToBarcode is reset to false and logged.

## 5. Sequence Summary

### RFID → Barcode (Test Mode)

- On INVENTORY_STOP_EVENT (in test mode):

  - Set bSwitchFromRfidToBarcode = true (log change)

  - Call subscribeRfidHardwareTriggerEvents(false)

  - Notify UI to scan barcode

  - Call testBarcode()

- In testBarcode():

  - Call subscribeRfidHardwareTriggerEvents(false)

  - Call setTriggerEnabled(false)

### Barcode → RFID

- Call setTriggerEnabled(true)

  - On success, call subscribeRfidHardwareTriggerEvents(true)

  - Set bSwitchFromRfidToBarcode = false (log change)

## 6. Logging

- Every state change of bRfidBusy and bSwitchFromRfidToBarcode is logged with the previous and new value, and the reason for the change.

- This provides a clear trace for debugging and understanding trigger state transitions.

## 7. Thread Safety

- Both flags are declared volatile to ensure visibility across threads.

- All configuration changes are protected by resourceLock to prevent race conditions.

## 8. Rationale

This design ensures robust, deadlock-free, and predictable trigger event handling when toggling between RFID and Barcode modes, especially in concurrent and test scenarios.